MINEGUARD-AI HARDWARE ASSEMBLY & SPECIFICATION MANUAL

Smart India Hackathon 2026 — Problem Statement SIH26025
Student Prototype Platform Specification & Construction Guide

STUDENT PROTOTYPE NOTICE:
MineGuard-AI is a student prototype platform designed toward a scalable field architecture. It does NOT possess mine certification, intrinsic safety ratings, environmental qualification, or field-validated collapse prediction accuracy. All pin mappings are marked PROVISIONAL pending exact board choice.

# 1. Project Hardware Overview

MineGuard-AI addresses surface subsidence caused by underground coal mining. 4 prototype sensor nodes (N1–N4) and 1 ESP32 gateway are deployed ON THE SURFACE ABOVE underground coal panels to measure surface inclination, relative displacement delta between surface points, surface vibration, and continuity line crack initiation.

# 2. Safety & Electrical Notes

Always disconnect USB power before modifying wiring. Verify 3.3V vs 5V pin operating levels on the ESP32. Do NOT apply 5V directly to ESP32 GPIO input pins.

# 3. Complete Bill of Materials (BOM)

| Item # | Component | Quantity | Est. Unit Cost (INR) | Purpose |
| --- | --- | --- | --- | --- |
| 1 | ESP32 Development Board (30-pin WROOM-32) | 5 (4 nodes + 1 GW) | ₹350 | Microcontroller & Wireless ESP-NOW Communication |
| 2 | MPU6050 6-DOF Accelerometer & Gyroscope | 4 | ₹120 | Surface tilt inclination & vibration RMS sensing |
| 3 | Linear Slider / String Potentiometer (10k) | 4 | ₹80 | Relative displacement delta between surface node pairs |
| 4 | Conductive Continuity Line / Break Wire | 4 | ₹15 | Prototype crack detector continuity bridge |
| 5 | Active Piezo Buzzer & Indicator LED | 4 | ₹25 | Local fail-safe threshold alarm trigger |
| 6 | Solderless Breadboards & Jumper Wires | 1 Set | ₹200 | Prototype circuit assembly |
| 7 | USB Micro/Type-C Cables | 2 | ₹100 | Gateway USB Web Serial power and data connection |

# 4. Exact Pin Mapping (PROVISIONAL / VERIFY WITH ACTUAL BOARD)

Pin definitions are centralized in hardware_config.h. The table below outlines the provisional pin configuration for ESP32-WROOM-32.

| Component | Component Pin | ESP32 GPIO | Electrical Purpose / Constraints |
| --- | --- | --- | --- |
| MPU6050 | SDA | GPIO 21 | I2C Data line (3.3V) |
| MPU6050 | SCL | GPIO 22 | I2C Clock line (3.3V) |
| String Potentiometer | Wiper Out | GPIO 34 (ADC1_CH6) | Analog displacement input (Input-Only, 0-3.3V) |
| Crack Detector | Continuity Return | GPIO 4 | Digital Input with Internal Pull-up (LOW=Intact, HIGH=Broken) |
| Local Buzzer | VCC Positive | GPIO 18 | Digital Output (High level 3.3V trigger) |
| ESP-NOW Gateway | USB Serial RX/TX | UART 0 (USB) | Web Serial JSON telemetry output to laptop |

# 5. Physical Placement & Surface Model Construction

Sensors are mounted on a controlled surface ground model above an underground coal extraction panel. The 4 nodes form a rectangle: Node N1 (top-left), Node N2 (top-right), Node N3 (bottom-right), Node N4 (bottom-left). String potentiometers span between N1-N2, N2-N3, N3-N4, N1-N4 to measure relative ground movement.

# 6. Sensor Calibration Procedure

1. MPU6050 Level Zeroing: Place node on horizontal surface; record X/Y zero offset.
2. Displacement Mechanism Zeroing: Extend string potentiometer to neutral baseline; set baseline ADC reference.
3. Crack Detector Test: Verify digital input pin reads LOW when conductive wire bridge is intact.

# 7. Step-by-Step SIH Demonstration Procedure

1. Connect ESP32 Gateway to laptop over USB cable.
2. Open MineGuard-AI web application in Chrome browser.
3. Click [Connect Hardware] -> Web Serial -> Select ESP32 Port.
4. Observe 4 surface nodes NORMAL baseline.
5. Apply surface tilt to Node N3 -> Web app displays WARNING.
6. Pull displacement mechanism between N3 and N4 -> Web app displays CRITICAL & Turf.js spatial risk boundary polygon.
7. Disconnect conductive wire on N4 -> Local buzzer triggers & web alert displays CRACK EVENT.
8. Toggle offline mode -> System stores readings to IndexedDB queue and syncs when reconnected.

# 8. Troubleshooting Guide

Issue: ESP32 Port Not Detected -> Ensure CP210x / CH340 USB driver is installed.
Issue: Web Serial Permission Denied -> Grant browser serial access when prompted.
Issue: MPU6050 Read Error -> Verify SDA/SCL pullup resistors and 3.3V power connection.